Working Procedure of
Line Following Robot

| Section - 2241026 | Group - 01 |
| --- | --- |
| Team Members | Regd No. |
| Dinanath Dash | 2241004161 |
| Himanshu Sekhar Dash | 2241004163 |
| Priyansu Nayak | 2241014108 |
| Ashutosh Raj | 2241018015 |
| Swarnabha Roy | 2241018192 |
| Subhojeet Sarkar | 2241019321 |

WORKING PROCEDURE:

- POWER ON AND SYSTEM INITIALIZATION

- Turn on the power switch. Voltage is supplied to Arduino, ESP module, sensors, and motor driver. Arduino initializes GPIO pins for sensor inputs and motor control outputs, and begins serial communication with ESP. ESP module boots up and creates WiFi Access Point with SSID: "Robot-Control" and Password: "12345678". ESP sends ready signal to Arduino via serial.

- WIFI CONNECTION SETUP

- Connect your Android phone/laptop to WiFi network "Robot-Control" using password "12345678".

- Android App Control: Install RobotController app from https://github.com/DinanathDash/RobotController/releases/latest/download/app-debug.apk. Open app (automatically connects to 192.168.4.1).

- Web Page Control: Open browser and go to http://192.168.4.1 (static control web page loads).

- AUTONOMOUS LINE-FOLLOWING MODE

- Sensor Reading: Arduino reads IR line sensors connected to digital pins. Sensors detect black/white line (HIGH/LOW signals).

- Control Logic: Arduino processes sensor data:

    - Both sensors OFF → Move Straight (both motors forward)

    - Left sensor ON, Right OFF → Turn Left (right motor forward, left stop/slow)

    - Right sensor ON, Left OFF → Turn Right (left motor forward, right stop/slow)

    - Both sensors ON → Stop or continue straight

- Motor Execution: Arduino sends PWM signals to motor driver. DC motors drive robot along line. Loop repeats continuously.

- MANUAL WIFI CONTROL MODE

- Command Initiation: Press buttons on app or web page (Forward, Backward, Left, Right, Stop, Mode Toggle).

- ESP Processing: App/web sends HTTP request to http://192.168.4.1/command?cmd=FORWARD. ESP parses command and sends serial data to Arduino (F=Forward, L=Left, etc.).

- Arduino Execution: Arduino receives serial data:

    - "F" → Both motors forward

    - "B" → Both motors reverse

    - "L" → Right motor forward, left stop

    - "R" → Left motor forward, right stop

    - "S" → Both motors stop

    - "AUTO" → Switch to autonomous mode

    - "MANUAL" → Switch to manual mode

- MODE SWITCHING

- Select AUTO or MANUAL on app/web page. ESP sends serial command to Arduino. Arduino switches operation mode.

- SAFE OPERATION

- Emergency Stop: Press STOP on app/web (immediate motor stop).

- Connection Loss: Arduino stops motors if no ESP communication.

- Power Off: Turn off power switch after stopping motors.