Practical Robotics Projects with Arduino

(CSE 4571)

Lab Assignment No – 08

Bluetooth Communication

Submission Date: _______________

| Branch: CSE Section: 2241026 |  |  |
| --- | --- | --- |
| Name | Registration No. | Signature |
| Dinanath Dash | 2241004161 |  |

Department of Computer Science and Engineering

Institute of Technical Education and Research (Faculty of Engineering)

Siksha 'O' Anusandhan (Deemed to be University)

Bhubaneswar, Odisha-751030.

Aim:

To interface an HC-05/HC-06 Bluetooth module with Arduino UNO for wireless serial communication between the Arduino and a Bluetooth-enabled device.

Objectives:

- To gain familiarity with the HC-05/HC-06 module and Arduino UNO serial communication.

- Learn the pin configuration (VCC, GND, TXD, RXD, EN/KEY) of the HC-05/HC-06 module.

- Understand the role of baud rate, pairing code, and serial communication protocol.

- To establish a basic wireless link between Arduino UNO and a smartphone/PC.

- Connect the HC-05/HC-06 module to the Arduino UNO (using SoftwareSerial).

- Write a simple Arduino sketch to send and receive data over Bluetooth.

- Pair the module with a smartphone/PC using a Bluetooth terminal app and verify successful two-way communication.

- To implement bidirectional communication and LED control.

- Build an external circuit with an LED connected to Arduino digital pin 6.

- Modify the Arduino program to turn the LED ON when the smartphone sends the character 1, and OFF when it sends 0.

- Send acknowledgment messages back to the smartphone (e.g., “LED ON” / “LED OFF”).

- To Evaluate communication performance.

- Test and record the effective communication range of the Bluetooth module.

- Measure response latency between sending a command (from phone) and Arduino execution.

- Pre-Lab Questionnaire:

- Name all the pins of the HC-05/HC-06 module and their functions.

- Why is a voltage divider required between Arduino TX and HC RX?

- What is the default baud rate of the HC-05/HC-06 module?

- Explain the difference between HC-05 and HC-06 modules.

- What is Software Serial in Arduino, and why is it used in this experiment?

- What is the purpose of pairing a Bluetooth device before communication?

- How does Arduino interpret commands sent from a smartphone?

- How can you verify if the HC-05/HC-06 module is powered and ready for pairing?

- List the factors that can affect Bluetooth communication range.

- Explain the role of the Serial Monitor in this experiment.

Answers to Pre-Lab Questions

Components/Equipment Required:

| Sl. No. | Name of the Component / Equipment | Specification | Quantity |
| --- | --- | --- | --- |
| 1) | Arduino UNO R3 | 16MHz | 1 |
| 2) | Arduino UNO cable | USB Type A to Micro-B | 1 |
| 3) | HC-05 / HC-06 Bluetooth Module | Bluetooth 2.0 SPP, 3.3V TTL logic, default baud rate 9600 bps | 1 |
| 4) | Resistors (carbon type) | 220Ω / 2.2kΩ/ 1 kΩ | 1 each |
| 5) | LED | Any colour of your choice | 1 |
| 6) | Breadboard | 840 Tie points | 1 |
| 7) | Smartphone with Bluetooth / PC with Bluetooth | Android/iOS device or Windows PC, for testing wireless serial communication | 1 |
| 8) | Bluetooth Terminal App | Android/iOS app like “Serial Bluetooth Terminal” or “Bluetooth Terminal | 1 |
| 9) | Jumper Wire | --------------------------- | As per requirement |

Objective 1

- To Gain familiarity with the HC-05/HC-06 module and Arduino UNO serial.

Circuit / Schematic Diagram

Figure 1: HC-05/HC-06 module Pinout

Observation

(Write how you have done the connection between the Bluetooth module and Arduino uno)

Objective 2

- To establish a basic wireless link between Arduino UNO and a smartphone/PC.

Circuit / Schematic Diagram

- Figure 2: Interface between Arduino UNO and HC-05/HC-06 module

Code

#include <SoftwareSerial.h>

SoftwareSerial BT(10, 11); // RX, TX

void setup() {

Serial.begin(9600);

BT.begin(9600);

Serial.println("Ready to communicate. Type on Serial Monitor or Bluetooth app.");

}

void loop() {

if (Serial.available()) {

char data = Serial.read();

BT.write(data);

}

if (BT.available()) {

char data = BT.read();

Serial.write(data);

}

}

Observation

Figure 3: Interface of Arduino UNO with HC-05/HC-06 module

Figure 4: Interface of Arduino UNO with HC-05/HC-06 module (Hardware)

Objective 3

- To implement bidirectional communication and LED control.

Circuit / Schematic Diagram

- Figure 5: Interface between Arduino UNO and HC-05/HC-06 module

Code

#include <SoftwareSerial.h>

SoftwareSerial BT(10, 11); // RX, TX

int led = 6;

void setup() {

pinMode(led, OUTPUT);

Serial.begin(9600);

BT.begin(9600);

Serial.println("Send '1' to turn ON LED, '0' to turn OFF.");

}

void loop() {

if (BT.available()) {

char cmd = BT.read();

Serial.print("Received: ");

Serial.println(cmd);

if (cmd == '1') {

digitalWrite(led, HIGH);

BT.println("LED ON");

Serial.println("LED turned ON");

}

else if (cmd == '0') {

digitalWrite(led, LOW);

BT.println("LED OFF");

Serial.println("LED turned OFF");

}

else {

BT.println("Invalid Command");

}

}

}

Observation

Figure 6: Interface HC-06 and Arduino UNO for bidirectional communication and LED control and confirmation in serial monitor window

Figure 7: Interface HC-06 and Arduino UNO for bidirectional communication and LED control and confirmation in serial monitor window (Hardware)

Objective 4

- To Evaluate communication performance.

Circuit / Schematic Diagram

- Figure 8: Establish connection with the Serial Monitor and Mobile Phone

Code

#include <SoftwareSerial.h>

SoftwareSerial BT(10, 11); // RX, TX

int led = 6;

void setup() {

pinMode(led, OUTPUT);

Serial.begin(9600);

BT.begin(9600);

Serial.println("Send '1' to turn ON LED, '0' to turn OFF.");

}

void loop() {

if (BT.available()) {

char cmd = BT.read();

Serial.print("Received: ");

Serial.println(cmd);

if (cmd == '1') {

digitalWrite(led, HIGH);

BT.println("LED ON");

Serial.println("LED turned ON");

}

else if (cmd == '0') {

digitalWrite(led, LOW);

BT.println("LED OFF");

Serial.println("LED turned OFF");

}

else {

BT.println("Invalid Command");

}

}

}

Observation

Figure 9: Test, record and Measure response latency  (Hardware)

Conclusion

Precautions

Post Experiment Questionnaire:

- How did you establish bidirectional communication between Arduino and a smartphone?

- Describe the steps to control an LED using Bluetooth commands.

- What was the maximum reliable communication range achieved during the experiment?

- How did you implement forwarding between Serial Monitor and Bluetooth?

- What challenges did you face while pairing the HC-05/HC-06 module?

- Explain how the Arduino code processes different commands (1 = ON, 0 = OFF).

- How would you modify the setup to control multiple LEDs with different commands?

- Compare the observed LED behavior when using Serial Monitor vs. smartphone commands.

Answers to Post-Lab Questions

| (Signature of the Faculty) | (Signature of the Student) |
| --- | --- |
|  | Name: |
| Date: | Registration No.: |
|  | Branch: |
|  | Section |